IT-SIMPLERA INSTITUTE

Network Administration Internship Program

WEEK 01 INTERNSHIP TASK REPORT Networking Fundamentals & Network Simulation Tools

| Submitted By | M Anees Zeb Khan |
| --- | --- |
| Internship Domain | Network Administration |
| Institute | IT-Simplera Institute |
| Supervisor | Jawad Qayum (Senior Network Administrator) |
| Submission Date | 28 June 2026 |
| Deadline | 3 July 2026 |
| Week | Week 01 |

Confidential – For Internal Assessment Purposes Only

# 1. Introduction

This report documents the completion of Week 01 of the Network Administration Internship at IT-Simplera Institute. The primary objective of this week was to gain foundational knowledge of computer networking concepts and to acquire practical, hands-on experience with two industry-standard network simulation platforms: Cisco Packet Tracer and GNS3.

Under the guidance of Senior Network Administrator Jawad Qayum, I studied core networking principles, installed and explored both simulation tools, designed Local Area Network (LAN) topologies, configured static IP addressing on virtual machines, and verified end-to-end connectivity using the ping command. This report presents a structured account of all concepts learned, activities performed, screenshots captured, and challenges encountered throughout the week.

# 2. Networking Concepts Learned

## 2.1  What is a Computer Network?

A computer network is a collection of interconnected devices (nodes) that communicate and share resources. Networks are categorized by geographic scale: LAN (Local Area Network), MAN (Metropolitan Area Network), and WAN (Wide Area Network). This week focused on LAN environments.

## 2.2  Network Devices

- Switch – A Layer 2 device that forwards frames based on MAC address tables, enabling multiple devices to communicate within the same network segment.

- Router – A Layer 3 device that routes IP packets between different networks.

- PC / End Device – The source or destination of network traffic.

- Hub – A legacy Layer 1 device that broadcasts traffic to all ports (not used in modern networks).

## 2.3  IP Addressing

IP addressing is the mechanism by which devices are uniquely identified on a network. This week, static IPv4 addressing was used:

- IP Address: A 32-bit identifier written in dotted-decimal notation (e.g., 192.168.1.1).

- Subnet Mask: Defines the network and host portions of an IP address (e.g., 255.255.255.0 for /24 networks).

- Default Gateway: The IP address of the router that traffic is forwarded to when the destination is outside the local subnet.

## 2.4  The OSI Model

The Open Systems Interconnection (OSI) model is a conceptual framework with 7 layers — Physical, Data Link, Network, Transport, Session, Presentation, and Application — that standardizes how different network protocols communicate. Understanding this model is foundational for troubleshooting and design.

## 2.5  The ping Command

The ping command uses ICMP (Internet Control Message Protocol) Echo Request/Reply messages to test reachability between two hosts. A successful ping confirms Layer 3 connectivity and proper IP configuration. Packet loss or timeouts indicate network or configuration issues.

# 3. Tools Installed & Overview

## 3.1  Cisco Packet Tracer

Cisco Packet Tracer is a free, beginner-friendly network simulation tool developed by Cisco Systems. It allows users to design, configure, and troubleshoot virtual network topologies using a drag-and-drop graphical interface without requiring physical hardware.

Key features include: a rich library of Cisco devices (routers, switches, PCs, servers), real-time and simulation modes, built-in terminal access, IP configuration GUI, and PDU (Packet Data Unit) tracking for visual packet analysis.

## 3.2  GNS3

GNS3 (Graphical Network Simulator-3) is a powerful, open-source network simulation platform used by professionals and certification candidates. Unlike Packet Tracer, GNS3 runs actual network operating system images (IOS, etc.) and supports VPCS (Virtual PC Simulator) for end-host emulation.

Key features include: support for real Cisco IOS, Juniper, and other device images; VPCS for lightweight PC simulation; telnet-based console access; integration with real hardware; and a topology canvas with detailed node management.

# 4. Cisco Packet Tracer – LAN Topology

## 4.1  Topology Design

A simple star-topology LAN was designed in Cisco Packet Tracer consisting of three PC-PT end devices (PC0, PC1, PC2) connected to a single 2960-24TT Layer 2 switch (Switch0). All devices were placed in the logical workspace.

Figure: Cisco Packet Tracer – Logical workspace with Switch0, PC0, PC1, PC2 placed

The switch (Cisco 2960-24TT) was selected from the Switches category in the device toolbar. Three PC-PT devices were selected from the End Devices category and placed on the canvas.

Figure: Cisco Packet Tracer – Switch added to workspace (2960-24TT)

Figure: Cisco Packet Tracer – End device selection toolbar (PC-PT highlighted)

## 4.2  Connecting Devices

Straight-through Ethernet cables were used to connect each PC to the switch using the auto-connect feature. After connecting the cables and waiting for STP to converge, all interface status indicators turned green, confirming active links.

Figure: Cisco Packet Tracer – Fully connected star topology with active green links

Figure: Cisco Packet Tracer – Complete LAN topology (PC0, PC1, PC2 connected to Switch0)

## 4.3  IP Address Configuration

Static IP addresses were assigned to each PC via Desktop > IP Configuration. The following address scheme was used:

| Device | IP Address | Subnet Mask |
| --- | --- | --- |
| PC0 | 192.168.1.1 | 255.255.255.0 |
| PC1 (shown as PC1 in config) | 192.168.1.2 | 255.255.255.0 |
| PC2 | 192.168.1.3 | 255.255.255.0 |

Figure: Cisco Packet Tracer – IP Configuration screen on PC1 (192.168.1.2 / 255.255.255.0)

Figure: Cisco Packet Tracer – PC Desktop tab showing IP Configuration, Command Prompt and other tools

## 4.4  Connectivity Verification – Ping Test

After configuring all IP addresses, connectivity was verified from PC0 using the Command Prompt (Desktop > Command Prompt). PC0 successfully pinged both PC1 (192.168.1.2) and PC2 (192.168.1.3) with 0% packet loss.

Figure: Cisco Packet Tracer – Ping from PC0 to PC1 (192.168.1.2) and PC2 (192.168.1.3) – 0% loss

# 5. GNS3 – LAN Topology

## 5.1  Project Setup & Topology Design

A new GNS3 project named "Week1_LAN" was created. Three VPCS (Virtual PC Simulator) nodes — PC1, PC2, PC3 — and one Ethernet Switch (Switch1) were added to the topology canvas. The project ran on GNS3 version 2.2.40.1 on Windows 64-bit.

Figure: GNS3 – Initial workspace: PC1, PC2, PC3 (VPCS) placed without connections

The Topology Summary panel on the right confirmed all three VPCS nodes (PC1 on telnet localhost:5000, PC2 on :5002, PC3 on :5004) and Switch1 were registered. All nodes showed green (active) status after starting.

Figure: GNS3 – Topology with Switch1, PC1, PC2, PC3 connected; Topology Summary panel visible

## 5.2  Connecting Devices

Ethernet links were drawn between each VPCS and Switch1. Green dot indicators on all links confirmed successful link-up after the topology was started (Play button). The connections were:

- PC1 → Switch1 (e0)

- PC2 → Switch1 (e1)

- PC3 → Switch1 (e2)

Figure: GNS3 – All VPCS nodes connected to Switch1 with active green link indicators

## 5.3  VPCS IP Address Configuration

Each VPCS console was accessed individually via the telnet localhost ports listed in Topology Summary. Static IP addresses were assigned using the VPCS "ip" command:

PC1> ip 192.168.1.1 255.255.255.0

PC2> ip 192.168.1.2 255.255.255.0

PC3> ip 192.168.1.3 255.255.255.0

During configuration of PC3, a duplicate address warning appeared: "PC1 : 192.168.1.3 255.255.255.0" – indicating that the system detected a possible conflict. However, since PC1 in VPCS was assigned .1 and the warning referred to a VPCS internal check, the configuration proceeded and connectivity was still verified successfully.

Figure: GNS3 – VPCS PC3 terminal: ip 192.168.1.3 assigned, duplicate-address check message shown

## 5.4  Connectivity Verification – Ping Test

From the PC3 VPCS console, a ping was issued to 192.168.1.1 (PC1). All 5 ICMP echo replies were received successfully with TTL=64 and response times ranging from 1.7ms to 2.0ms, confirming full LAN connectivity.

Figure: GNS3 – PC3 pinging 192.168.1.1 (PC1): 5/5 replies received – 0% packet loss

# 6. Comparison: Cisco Packet Tracer vs GNS3

| Feature | Cisco Packet Tracer | GNS3 |
| --- | --- | --- |
| Target Users | Beginners, Students | Professionals, CCIE candidates |
| Device Emulation | Simulated (virtual) | Real IOS images |
| End Hosts | PC-PT (GUI-based) | VPCS (CLI-based) |
| Console Access | Built-in terminal | Telnet via localhost ports |
| Resource Usage | Low | Higher (runs real OS) |
| IP Config Method | GUI (IP Configuration) | CLI (ip command) |
| Cost | Free (Cisco NetAcad) | Free (open source) |

# 7. Challenges Faced

- Duplicate Address Warning in GNS3: When assigning 192.168.1.3 to PC3, VPCS displayed a warning referencing PC1. This was understood as an internal VPCS ARP check rather than a true conflict; the ping test confirmed connectivity was intact.

- GNS3 Disk Space Warning: GNS3 reported that only 3.60% or less free disk space was detected on the project drive. This required cleanup of temporary files before proceeding with the project.

- STP Convergence Delay in Cisco Packet Tracer: After connecting all devices, the switch interfaces initially showed orange indicators. It was necessary to wait ~30 seconds for Spanning Tree Protocol to converge before the links turned green and pings succeeded.

- Understanding VPCS CLI vs GUI: Transitioning from the graphical IP Configuration tool in Cisco Packet Tracer to the VPCS command-line interface in GNS3 required careful attention to syntax and command structure.

# 8. Conclusion

This first week of the Network Administration Internship at IT-Simplera Institute provided a solid foundation in computer networking principles and practical experience with two leading simulation platforms. By successfully designing, configuring, and verifying a three-host LAN topology in both Cisco Packet Tracer and GNS3, I demonstrated the ability to apply IP addressing concepts, work with Layer 2 switching, and use diagnostic tools to confirm network connectivity.

The experience highlighted the complementary nature of both tools — Cisco Packet Tracer for rapid visual learning and GNS3 for realistic, production-grade emulation. The challenges encountered, including the VPCS duplicate-address check and STP convergence wait, deepened my understanding of real-world networking behaviour.

I am confident that the skills and knowledge gained this week form a strong base for the more advanced topics and configurations that will be covered in subsequent weeks of the internship.

M Anees Zeb Khan

Network Administration Intern  |  IT-Simplera Institute  |  June 2026